Practical thingspeak

#include <WiFi.h>

#include "DHT.h"

#include "HTTPClient.h"

// ====== WiFi Credentials ======

const char* ssid = "ENTC04";         // your WiFi name

const char* password = "$inhgad@2122";   // your WiFi password

// ====== ThingSpeak ======

String apiKey = "5UP8HD2GGD9E0CWG";  // your ThingSpeak Write API key

const char* server = "http://api.thingspeak.com/update";

// ====== DHT11 Setup ======

#define DHTPIN 4       // GPIO pin where DHT11 data pin is connected

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(115200);

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\n✅ Connected to WiFi");

dht.begin();

}

void loop() {

float h = dht.readHumidity();

float t = dht.readTemperature(); // Celsius

if (isnan(h) || isnan(t)) {

Serial.println("❌ Failed to read from DHT11 sensor!");

return;

}

Serial.print("🌡 Temperature: ");

Serial.print(t);

Serial.print(" °C   💧 Humidity: ");

Serial.print(h);

Serial.println(" %");

if (WiFi.status() == WL_CONNECTED) {

HTTPClient http;

String url = String(server) + "?api_key=" + apiKey + "&field1=" + String(t) + "&field2=" + String(h);

http.begin(url);

int httpCode = http.GET();

if (httpCode > 0) {

Serial.println("✅ Data sent to ThingSpeak successfully!");

} else {

Serial.println("❌ Error sending data");

}

http.end();

} else {

Serial.println("⚠ WiFi disconnected! Trying to reconnect...");

WiFi.begin(ssid, password);

}

delay(20000); // ThingSpeak minimum update interval = 15 seconds

}

naikchetan666@gmail.com

ChetanNaik@2003